P

Procedure & Subject Matter Protocol

Table of Contents

1.	Introduction	4

2.	Connection Mode	4

3.	Link Test Message	4

4.	Procedures of Data transaction	5

5.	Message Format	5

6.	Message List	8

7.	Message Detail	11

8.	Normal Process Scenario	25

9.	Abnormal Process Scenario	29

| Date | Reason/History | Revision | By whom | Signature |
| --- | --- | --- | --- | --- |
| 2019/11/12 | Create | 1.0 | David Ma |  |
| 2019/11/18 | SOH Report added in S82 | 1.1 | David Ma |  |
| 2021/05/13 | Move Change added in P85 | 1.2 | David Ma |  |
| 2021/05/21 | 修正 S82 錯誤 | 1.3 | David Ma |  |
| 2021/06/30 | P61新增2+4、6碼AlarmCode | 1.4 | David Ma |  |
| 2021/09/02 | P25更新四槽位欄位 | 1.5 | David Ma |  |
| 2021/09/06 | 修正 P43 錯誤 | 1.6 | David Ma |  |
| 2021/09/11 | 新增P87、S88移除P19、修正P17、S18、S82 | 1.7 | David Ma |  |
| 2022/03/03 | 新增P89、S90 | 1.8 | David Ma |  |
| 2022/05/17 | 修改PS Protocol P65 | 1.9 | David Ma |  |
| 2022/07/15 | 新增Map Change、ABCS Control | 2.0 | David Ma |  |
| 2022/08/24 | 修改Map Change PS Protocol P31 | 2.1 | David Ma |  |
| 2022/11/03 | 移除 PS Protocol P67，修改P89、S90敘述 | 2.2 | David Ma |  |
| 2023/12/19 | 新增PS Protocol P51 Reach and Stop | 2.3 | David Ma |  |
| 2023/05/23 | 新增PS Protocol P61 2: reset warning 3: set warning | 2.4 | David Ma |  |
| 2023/05/26 | 新版P45指令 | 2.5 | David Ma |  |
| 2023/07/20 | 修改PS Protocol P27 | 2.6 | David Ma |  |
| 2024/01/03 | 修改P25指令 | 3.0 | Mason Lin |  |
| 2024/09/20 | 新增P19指令 4.1 |  | Jason Chi |  |
| 2024/10/23 | 新增P39指令，修改P45指令 5.0 |  | Mike Lin |  |

Revision History

- 目的 Purpose：定義 TCP/IP上訊息傳遞的通訊控制介面。

- 適用範圍 Scope：這份規格文件是針對捷螺內部或與供應商之間，關於系統間透過TCP/IP傳遞訊息的通訊控制規範，適用範圍涵蓋：單機或複合型式的主製程設備、檢測設備(測試設備)、小型工具機及傳送設備等不同控制軟體間的通訊使用。

- 名詞定義 Terms Definition：無

- 作業流程與內容 Procedure & Subject Matter：

Notice:

- 此規範不是最終版本，可依機台特性修改。

This specification is not the final version and can be modified depend on the equipment characteristic.

- 機台廠商須給免費解決所有軟體 bug 於捷螺公司或客戶端。

The equipment provider has to deliver bug-free software to Gyro before it is tested at Gyro  or customer site.

- 如果在測試期間有任何 bug,不論是否與規格相符或為軟體本身 bug，捷螺保留最後規格的決定權，且不須負任何費用給廠商。

If there is any bug during the testing stage, whether it is specification discrepancy or software bug, Gyro  reserves the final right to perform the specification change without any charge by vendor.

- 於驗收測試後，廠商必須提供 3 年的保證服務。

After the acceptance test the vendor have to provide three-year warranty service.

- Introduction

PS Protocol is a simple protocol using at communication via TCP/IP. It’s similar to HSMS + SECS II standard but easy to implement. It directly uses string transferring to byte array than send out to the other side.

- Connection Mode

Active Side: TCP Client

Passive side: TCP Server

- Link Test Message

P00: Link Test Request

S00: Link Test Reply

Both Active/Passive side can send P00 to the other side and receiver must reply S00 immediately. The side who send P00 need to check T6 timeout.

- Procedures of Data transaction

- Message Format

The message format composes message length, header, data and checksum. Size of message length is 1 byte, header is 3 bytes, system bytes are 4 bytes, and data is other bytes.

Block — header plus up to 125 bytes of data.

Message Lengths — The unsigned integer value of the first byte sent after receipt of EOT is the length of the block being sent. The length includes the bytes sent after the length byte, excluding the 2 bytes of the checksum. The maximum block length allowed by message is 224 bytes, and the minimum is 3 bytes.

System Bytes — The system bytes is unique. The system bytes of secondary message is copy from primary message.

Header —The operation of all communications functions above the message transfer protocol is linked to information contained in a 3-byte data element called the header. The header is always the first 3 bytes of every block sent by the message transfer protocol.

Primary Message (hereinafter called as “P”)— A primary message is defined as any odd numbered message.

Secondary Message (hereinafter called as “S”) — A secondary message is defined as any even numbered message.

The header and data information type is character of ASCII, message length and system bytes are Hex of ASCII (see Figure 4).

Reply Timeout, T3 — The reply timeout, T3, limits the time between sending primary message and receiving secondary message.

Link Test Reply Timeout, T6 — The link test reply timeout, T6, limits the time between sending P00 and receiving S00.

Message lengths 7	6	5	4	3	2	1	0

| 04 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Header

| P | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 1 |

Data

| 3 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 |
| 9 | 0 | 0 | 1 | 1 | 1 | 0 | 0 | 1 |
| 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 |

System Bytes

| 02 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 02 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |
| 02 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |
| 17 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 |

Primary Message Sample

Message lengths 7	6	5	4	3	2	1	0

| 00 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Header

| S | 0 | 1 | 0 | 1 | 0 | 0 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 0 | 0 | 1 | 1 | 0 | 1 | 0 | 0 |
| 2 | 0 | 0 | 1 | 1 | 0 | 1 | 0 | 0 |

Data

System Bytes

| 02 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 02 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |
| 02 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |
| 17 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 |

Secondary Message Sample

- Message List

| Message Classification | Message ID | Message Direction | Description |
| --- | --- | --- | --- |
| ON-Line/ OFF-Line Request | P11 | TSC 🡸Loc | Try Auto Request |
|  | S12 | TSC 🡺Loc | Try Auto Request Ack. |
|  | P13 | TSC 🡸Loc | Set Manual Request |
|  | S14 | TSC 🡺Loc | Set Manual Request Ack. |
|  | P15 | TSC 🡺Loc | Date and Time Setting |
|  | S16 | TSC 🡸Loc | Date and Time Setting Ack. |
|  | P17 | TSC🡸🡺Loc | Version Info Request |
|  | S18 | TSC🡸🡺Loc | Version Info Request Ack. |
|  | S20 | TSC 🡸Loc | WIFI Signal Strength Info Ack. |
|  | P21 | TSC 🡸Loc | Move Status Report |
| Passive and Active Status Report | S22 | TSC 🡺Loc | Move Status Report Ack. |
|  | P23 | TSC 🡸Loc | Robot Status Report |
|  | S24 | TSC 🡺Loc | Robot Status Report Ack. |
|  | P25 | TSC 🡸Loc | Slot status report |
|  | S26 | TSC 🡺Loc | Slot status report Ack |
|  | P27 | TSC 🡸Loc | Cassette Status Rename Request |
|  | S28 | TSC 🡺Loc | Cassette Status Rename Request Ack. |
|  | P29 | TSC 🡸Loc | Charge Status Report |
|  | S30 | TSC 🡺Loc | Charge Status Report Ack. |
|  | P31 | TSC 🡺Loc | Status Report Request |
|  | S32 | TSC 🡸Loc | Status Report Request Ack. |
|  | P33 | TSC 🡺Loc | Position Report Request |
|  | S34 | TSC 🡸Loc | Position Report Request Ack. |
|  | P35 | TSC 🡺Loc | Battery Status Request |
|  | S36 | TSC 🡸Loc | Battery Status Request Ack. |
|  | P37 | TSC 🡺Loc | Command info report |
|  | S38 | TSC 🡸Loc | Command info report Ack |
|  | P39 | TSC 🡺Loc | Current command info report |

| Message Classification | Message ID | Message Direction | Description |
| --- | --- | --- | --- |
| Command Request | P41 | TSC 🡺Loc | Move Command |
|  | S42 | TSC 🡸Loc | Move Command Ack |
|  | P43 | TSC 🡺Loc | Move Append Arrival Report |
|  | S44 | TSC 🡸Loc | Move Append Arrival Report Ack. |
|  | P45 | TSC 🡺Loc | Robot Command Request |
|  | S46 | TSC 🡸Loc | Robot Command Request Ack. |
|  | P47 | TSC 🡺Loc | Charge Request |
|  | S48 | TSC 🡸Loc | Charge Request Ack. |
|  | P49 | TSC 🡺Loc | Clear Robot Command Request |
|  | S50 | TSC 🡸Loc | Clear Robot Command Request Ack. |
|  | P51 | TSC 🡺Loc | Stop Request |
|  | S52 | TSC 🡸Loc | Stop Request Ack. |
|  | P53 | TSC 🡸Loc | Robot command result report |
|  | S54 | TSC 🡸Loc | Robot command result report Ack |
|  | P81 | TSC 🡺Loc | AGV info request |
|  | S82 | TSC 🡸Loc | AGV info request Ack |
|  | P83 | TSC 🡺Loc | Robot Command |
|  | S84 | TSC 🡸Loc | RobotCommand Ack |
|  | P85 | TSC 🡺Loc | Move Change Request |
|  | S86 | TSC 🡸Loc | Move Change Request Ack |
|  | P87 | TSC 🡸Loc | Robot Command |
|  | S88 | TSC 🡸Loc | AGV info request Ack |
|  | P89 | TSC 🡺Loc | Erack Placement Signal |
|  | S90 | TSC 🡸Loc | Erack Placement Signal Ack. |

| Message Classification | Message ID | Message Direction | Description |
| --- | --- | --- | --- |
| Alarm Set and Reset Request | P61 | TSC🡸🡺Loc | Alarm Report |
|  | S62 | TSC🡸🡺Loc | Alarm Report Ack. |
|  | P63 | TSC🡸🡺Loc | Reset All Alarm Request |
|  | S64 | TSC🡸🡺Loc | Reset All Alarm Request Ack. |
|  | P65 | TSC 🡸Loc | Move Obstacles |
|  | S66 | TSC 🡺Loc | Move Obstacles Ack. |

| Message Classification | Message ID | Message Direction | Description |
| --- | --- | --- | --- |
| MAP change | P91 | TSC🡺Loc | 回報目前地圖 |
|  | S92 | TSC🡸Loc | 回報目前地圖確認 |
|  | P93 | TSC🡺Loc | 更換地圖 |
|  | S94 | TSC🡸Loc | 更換地圖確認 |
|  | P95 | TSC 🡸Loc | 更換地圖狀態 |
|  | S96 | TSC 🡺Loc | 更換地圖狀態確認 |

| Message Classification | Message ID | Message Direction | Description |
| --- | --- | --- | --- |
| ABCS Control | P71 | TSC🡺Loc | 開始電池交換 |
|  | S72 | TSC🡸Loc | 回報狀態 |
|  | P73 | TSC🡺Loc | 交換成功、失敗 |
|  | S74 | TSC🡸Loc | 收到 |
|  | P75 | TSC 🡸Loc | 電池交換狀態 |
|  | S76 | TSC 🡺Loc | 收到 |

| Message Classification | Message ID | Message Direction | Description |
| --- | --- | --- | --- |
| Event Report | P19 | TSC 🡸Loc | 事件回報 |
|  | S20 | TSC 🡺Loc | 收到 |

- Message Detail

| P00 | Contents: Link Test Request | Direction |
| --- | --- | --- |
|  | Structure：Header only | TSC 🡸🡺 Loc |
| S00 | Contents: Link Test Request Ack. | Direction |
|  | Structure：Header only | TSC🡸🡺Loc |
| P11 | Contents: Try Auto Request | Direction |
|  | Structure：Header only | TSC 🡸Loc |
| S12 | Contents: Try Auto Request Ack. | Direction |
|  | Structure：Header only | TSC 🡺Loc |
| P13 | Contents: Set Manual Request | Direction |
|  | Structure：Header only | TSC 🡸Loc |
| S14 | Contents: Set Manual Request Ack. | Direction |
|  | Structure：Header only | TSC 🡺Loc |
| P15 | Contents: Date and Time Setting | Direction |
|  | Structure：14 Bytes YYYYMMDDHHMMSS E.g. Data = 20200304153152 | TSC 🡺Loc |
| P16 | Contents: Date and Time Setting Ack. | Direction |
|  | Structure：1 Bytes 0 = Date and Time Setting Accepted | TSC 🡸Loc |

| P17 | Contents: Version Info Request | Direction |
| --- | --- | --- |
|  | Structure：Header only | TSC🡸🡺Loc |
| S18 | Contents: Version Info Request Ack. | Direction |
|  | Structure：22 Bytes 15 Byte : Sw + Software Version 7 Byte : Sp + Spec Version E.g. Data = Sw2020.04.22.01Sp2.3 | TSC🡸🡺Loc |
| P19 | Contents: Event Report | Direction |
|  | Structure：1 Bytes 0 = Retry action | TSC🡸Loc |
| S20 | Contents: Event Report Ack. | Direction |
|  | Structure：Header only | TSC 🡺Loc |

| P21 | Contents: Move Status Report | Direction |
| --- | --- | --- |
|  | Structure：4 Bytes 1 Byte = 0:Idle, 1:Working, 2:Pausing, 3:Pause, 4:Stoping, 5:Block, 6:Error 2~4 Byte = Head Direction (0 ~ 359) E.g. AGVL want to update move state is Idle and head angle is 0. Data = 0000 E.g. AGVL want to update move state is Error and head angle is 90. Data = 6090 | TSC 🡸Loc |
| S22 | Contents: Move Status Report Ack. | Direction |
|  | Structure：Header only | TSC 🡺Loc |
| P23 | Contents: Robot Status Report | Direction |
|  | Structure：2 Bytes - Byte = Status, 0:Idle, 1:Busy, 2:Error - Byte = Home, 0:Not Home, 1:Home E.g. AGVL want to update robot state is Idle and robot is home. Data = 01 E.g. AGVL want to update robot state is Error and robot is not home. Data = 20 | TSC 🡸Loc |
| S24 | Contents: Robot Status Report Ack. | Direction |
|  | Structure：Header only | TSC 🡺Loc |

| P25 | Contents: Cassette Status Report | Direction |
| --- | --- | --- |
|  | Structure：N Byte 2 Byte = AGV槽位編號，01、02、03、04、......10、11...... - Byte = 0:Empty, 1:Loading, 2: ReadIdFail ,3: PositionError 3~N Byte = Cassette ID / Empty / ReadIdFail / PositionError 範例: AGVL第一槽位回報無Cassette Data = 010 AGVL 第二槽位回報Cassette ID (ABC) Data = 021ABC AGVL 第三槽位回報讀取失敗 Data = 032ReadIdFail or 032 AGVL第四槽位回報放置位置異常(PS/PL Sensor狀態異常) Data = 043PositionError | TSC 🡸Loc |
| S26 | Contents: Cassette Status Report Ack. | Direction |
|  | Structure：Header only | TSC 🡺Loc |

| P27 | Contents: Cassette Status Rename Request | Direction |
| --- | --- | --- |
|  | Structure：N Bytes 2 Byte = Vehicle carrier slot number 3~N Byte = Cassette ID E.g. TSC want to rename 01 slot cassette ID into “ABC”. Data = 01ABC E.g. TSC want to rename 12 slot cassette ID into “Error”. Data = 12Error | TSC 🡺Loc |
| S28 | Contents: Cassette Status Rename Request Ack. | Direction |
|  | Structure：Header only | TSC 🡸Loc |
| P29 | Contents: Charge Status Report | Direction |
|  | Structure：1 Bytes 0 = Off, Coupler back to home 1 = On, Since PIO start E.g. AGVL want to update battery is charging or has start charge PIO. Data = 1 E.g. AGVL want to update battery is not charging and has end charge PIO. Data = 0 | TSC 🡸Loc |
| S30 | Contents: Charge Status Report Ack. | Direction |
|  | Structure：Header only | TSC 🡺Loc |

| P31 | Contents: Status Report Request | Direction |
| --- | --- | --- |
|  | Structure：1 Bytes - = All Status - = Move Status - = Robot Status - = Cassette Status - = Charge Status - = 查詢地圖 E.g. TSC want to update all status. Data = 0 E.g. TSC want to update charge status. Data = 4 | TSC 🡺Loc |
| S32 | Contents: Status Report Request Ack. | Direction |
|  | Structure：Header only | TSC 🡸Loc |

| P33 | Contents: Position Report Request | Direction |
| --- | --- | --- |
|  | Structure：Header only | TSC 🡺Loc |
| S34 | Contents: Position Report Request Ack. | Direction |
|  | Structure：29 Bytes - Byte = 0:Idle, 1:Working, 2:Pausing, 3:Pause, 4:Stoping, 5:Stop, 6:Error - Byte = Position X signature, P:Positive,N:Negitive 3~10 Byte = Position X, 11 Byte = Position Y signature, P:Positive,N:Negitive 12~19 Byte = Position Y 20~22 Byte = Head Direction (0 ~ 359) 23~25 Byte = Moving Direction (0 ~ 359) 26~29 Byte = Move Speed E.g. AGVL want to update position is Idle at (12345678,-98765432) with head angle is 0 move direction and speed are 0. Data = 0P12345678N987654320000000000 E.g. AGVL want to update vehicle is move pass position (12345678,-98765432) with head angle is 90 move direction is 0 and speed is 1500. Data = 1P12345678N987654320900001500 | TSC 🡸Loc |

| P35 | Contents: Battery Status Request | Direction |
| --- | --- | --- |
|  | Structure：Header only | TSC 🡺Loc |
| S36 | Contents: Battery Status Request Ack. | Direction |
|  | Structure： 10 Byte - Byte = Percentage - Byte = Voltage 3 Byte = Temperature E.g. AGVL want to update battery state with Percentage is 72 , Voltage is 54.23,and Temperature is 40, Data = 0725423040 | TSC 🡸Loc |
| P37 | Contents: Set Command Info | Direction |
|  | Structure：N Bytes 1 Byte : Command Step, 0 = Begin, 1 = End. First <> : Command ID (Could be empty if end) Second <> : From Port Name (Could be empty) Third <> : To Port Name (Could be empty if end) Fourth <> : Cassette ID (Could be empty) Fifth <> : Lot ID (Could be empty) E.g. TSC want to update AGVL begin a transfer command with Command ID is Cmd001, From Port Num is 10007, To Port Num is 10005, using Buffer1, Lot ID = 001, and CST ID is CST002. P390<Cmd001><BUFFER1><10005><CST002><001> | TSC 🡺Loc |
| S38 | Contents: Set Command Info Ack. | Direction |
|  | Structure：1 Bytes 0 = NG, 1 = OK. | TSC 🡸Loc |
| P39 | Contents: Set Current Command Info | Direction |
|  | Structure：N Bytes 1 Byte : Command Step, 0 = Begin, 1 = End. First <> : Command ID (Could be empty if end) Second <> : From Port Name (Could be empty) Third <> : To Port Name (Could be empty if end) Fourth <> : Cassette ID (Could be empty) Fifth <> : Lot ID (Could be empty) E.g. TSC want to update AGVL begin a transfer command with Command ID is Cmd001, From Port Num is 10007, To Port Num is 10005, using Buffer1, Lot ID = 001, and CST ID is CST002. P390<Cmd001><BUFFER1><10005><CST002><001> | TSC 🡺Loc |
| S40 | Contents: Set Current Command Info Ack. | Direction |
|  | Structure：1 Bytes 0 = NG, 1 = OK. | TSC 🡸Loc |

| P41 | Contents: Move Append Request | Direction |
| --- | --- | --- |
|  | Structure：28 Bytes 1Byte: 0:無需退站進站，1:到站點後執行二次定位，2:在站點執行退站 2~10Byte: P(正)N(負)P00001234為上報Routemap X:1.234，若是N00001234 X:-1.234，取小數點三位 11~19Byte: P(正)N(負)P00001234為上報Routemap Y:5.678，若是N00001234 Y:-5.678，取小數點三位 20~22Byte: 車頭方向單位為度，通常為0、90、180、270度，注意不可有負數 23~26Byte: 速度限制:500 設定值/1000，500/1000=0.5 ，若當前最高車速為1m/s 1m/0.5 = 0.5m/s 27Byte: 選擇開啟AGV槽位 0 : 無，1 : 左開蓋，2 : 右開蓋，3 : 左右開蓋 28Byte = Keep 或 GO ，Keep時該命令不會執行，AGV只會執行GO 29Byte = 是否允許自由避障 0: 不可，1:可 30~NByte = 標示此為一段路徑中的第幾號。以"<"開頭，">"結尾。以idx/total表示此段路徑有total個點，此命令為第idx個。若收移動命令的過程中有跳號則會整筆命令拒絕。 範例: TSC請求AGVL先退站並移動到(X:1.234,Y:5.678)並且到站時角度為90度，速度限制為500/1000=0.5，當前AGV最高速*0.5，且不開蓋、會執行該命令，此路段不開放自由避障，目前為第1筆，總共有20筆 命令 = 2P00001234P0000567809005000G0<1/20> | TSC 🡺Loc |
| S42 | Contents: Move Append Request Ack. | Direction |
|  | Structure：1 Bytes 0 = NG, 1 = OK.. | TSC 🡸Loc |
| P43 | Contents: Move Append Arrival Report | Direction |
|  | Structure：29 Bytes - Byte = Is arrival or not, 0:Fail(Move Stop), 1:Arrival, 2:EndArrival - Byte = Position X signature, P:Positive,N:Negitive 3~10 Byte = Position X, 11 Byte = Position Y signature, P:Positive,N:Negitive 12~19 Byte = Position Y 20~22 Byte = Head Direction 23~25 Byte = Moving Direction 26~29 Byte =Speed 29~N Byte = Station ID E.g. AGVL arrive at (1234,-5678), arrival style is End, vehicle Head Angle at (1234,-5678) is 90 degree, Moving Driection is 90 degree, arrival speed is 0 , Station ID is ABC. Data = 2P00001234N000056780900900000ABC | TSC 🡸Loc |

| S44 | Contents: Move Append Arrival Report Ack. | Direction |
| --- | --- | --- |
|  | Structure：Header only | TSC 🡺Loc |
| P45 | Contents: Robot Command Request | Direction |
|  | Structure：24 Bytes <1st>: 0:無使用PIO(E84)，1:AGV左PIO(E84)，2:AGV右PIO(E84)，2:AGV前PIO(E84) <2nd>: E84 CS訊號，數字0到30 <3rd>: 0-沒有後續交握，1-有後續交握 <4th>: Carrier Type，數字 <5th>:可為BUFFER(車上儲位)，或是station ID，手臂取貨點 <6th>: 取貨點Port Number，數字 <7th>:可為BUFFER(車上儲位)，或是station ID，手臂放貨點 <8th>: 放貨點Port Number，數字1 <9th>: Extra info，內容為json格式 範例: 命令: P45<0><0><0><1><BUFFER><1><pErack001_1><2><{}> 在pErack001_1這站，從BUFFER1上到port2，carrier為第1種，不需要E84交握，無額外資訊 命令: P45<1><0><1><2><pErack001_1><1><BUFFER><1><{"height": "10"}> 在pP001這站，從port1下到BUFFER1，用左邊的sensor以CS_0做交握，carrier為第2種，後續有其他手臂動作，高度為10 命令: P45<2><1><0><3><BUFFER><1><pP001><2><{}> 在pP001這站，從BUFFER1上到port2，用右邊的sensor以CS_1做交握，carrier為第3種，後續無其他手臂動作，無額外資訊 | TSC 🡺Loc |
| S46 | Contents: Robot Command Request Ack. | Direction |
|  | Structure：1 Byte 0 = OK, 1 = From Port is Empty, 2 = To Port is Full. | TSC 🡸Loc |
| P47 | Contents: Charge Request | Direction |
|  | Structure：1 Bytes 1 Byte = Direction, 0:DisCharge, 1:Left, 2:Right. E.g. TSC request AGVL to Start charge with Right Side PIO. Data = 2 E.g. TSC request AGVL to Stop charge. Data = 0 | TSC 🡺Loc |

| S48 | Contents: Charge Request Ack. | Direction |
| --- | --- | --- |
|  | Structure：Header only | TSC 🡸Loc |
| P49 | Contents: Clear Robot Command Request | Direction |
|  | Structure：Header only | TSC 🡺Loc |
| S50 | Contents: Clear Robot Command Request Ack. | Direction |
|  | Structure：1 Bytes 0 = NG, 1 = OK.. | TSC 🡸Loc |
| P51 | Contents: Stop Request | Direction |
|  | Structure：1 Bytes 0 = Pause, 1 = Resume 2 = Stop 3 = Reach and stop | TSC 🡺Loc |
| S52 | Contents: Stop Request Ack. | Direction |
|  | Structure：1 Bytes 0 = NG, 1 = OK. | TSC 🡸Loc |
| P53 | Contents: Robot Command Finished Report | Direction |
|  | Structure：15 Bytes Finished = Robot Command Finished InterlockError = Interlock Error RobotError = Robot Error E.g. AGVL want to report robot command is well done. Data = Finished E.g. AGVL want to report robot command is fail at PIO before robot working. Data = InterlockError | TSC 🡸Loc |
| S54 | Contents: Robot Command Finished Report Ack. | Direction |
|  | Structure：Header only | TSC 🡺Loc |

| P61 | Contents: Alarm Report | Direction |
| --- | --- | --- |
|  | Structure：9 Bytes First Byte = 0:復歸(對指定ErrorCode進行復歸), 1:設定(產生指定ErrorCode) 3:復歸(對指定WarningCode進行復歸),4:設定(產生指定WarningCode) 2~6 Byte = Error Code (2+4數字16進制)，或是(2英文+4數字16進制) 2~9 Byte = Error Code(K21) 範例AGVL 對TSC上報Alarm 2~3 Byte: 10:TM Robot原廠，11:Routemap，12:E84，13:TCP_Bridge，14:TM自定義，15:BS，16:Encoder，17:Canbus，18:RFID，19:槽位門， 20:AGV零件。 命令 :110123456 若是清除該Alarm 命令 : 010123456 範例: TSC 對AGVL下令Alarm 使用8碼AlarmCode 2~3 Byte: 00:TSC 命令 : 100654321 若是清除該Alarm 命令 : 000654321 範例AGVL 對TSC上報Alarm使用6碼或4+2碼AlarmCode 2~3 Byte TM or 10 命令: 1TM6543 1106543 詳細異常列表，請參閱異常列表排除 | TSC🡸🡺Loc |
| S62 | Contents: Alarm Report Ack. | Direction |
|  | Structure：Header only | TSC🡸🡺Loc |
| P63 | Contents: Reset All Alarm Request | Direction |
|  | Structure：Header only | TSC🡸🡺Loc |
| S64 | Contents: Reset All Alarm Request Ack. | Direction |
|  | Structure：Header only | TSC🡸🡺Loc |
| P65 | Contents: Move Obstacles | Direction |
|  | Structure：1 Bytes 0 = Off, 1 = On. E.g. AGVL update obstacles info to TSC . Date = 1 E.g. AGVL detected obstacles info to TSC . Date = 0 | TSC 🡸Loc |
| S66 | Contents: Move Obstacles Ack | Direction |
|  | Structure：Header only | TSC 🡺Loc |

| P71 |  | Contents: 電池交換命令 | Direction |
| --- | --- | --- | --- |
|  |  | Structure：Header only | TSC🡺Loc |
| S72 |  | Contents: 電池交換命令確認 | Direction |
|  |  | Structure： 1 Byte = 0:NG, 1:OK | TSC🡸Loc |
| P73 |  | Contents: 電池交換結果 | Direction |
|  |  | Structure： 1 Byte = 0:交換完成, 1:交換失敗 | TSC🡸Loc |
| S74 | Contents: 電池交換結果確認 |  | Direction |
|  | Structure：Header only |  | TSC🡺Loc |
| P75 | Contents: 電池交換狀態 |  | Direction |
|  | Structure： 1 Byte = 0:不在交換1:交換中 |  | TSC🡸Loc |
| S76 | Contents: 電池交換狀態確認 |  | Direction |
|  | Structure：Header only |  | TSC🡺Loc |

|  | P81 |  | Contents: AGV info request |  | Direction |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Structure：Header only |  | TSC🡺Loc |  |
| S82 |  | Contents: AGV info request Ack |  | Direction |  |
|  |  | Structure： byte 1: AGV moving status. 0: Idle 1: Moving 3: Paused 6: Error byte 2: Sign of AGV x coordinate. P: Positive N: Negative byte 3~10: AGV x coordinate (without sign) in milimeter. byte 11: Sign of AGV y coordinate. P: Positive N: Negative byte 12~19: AGV y coordinate (without sign) in milimeter. byte 20~22: AGV head angle in degrees(from 0 to 360). byte 23~25: AGV moving direction in degrees(from 0 to 360). byte 26~29: AGV moving speed in milimeter. byte 30: AGV robot status. 0: Idle 1: Busy 2: Error byte 31: AGV robot at home point or not. 0: Robot is not at home point 1: Robot is at home point byte 32~34: AGV battery SOC(from 0 to 100). byte 35~38: AGV battery voltage in V. byte 39~41: AGV battery temperature in Celsius. byte 42~44: AGV battery SOH(from 0 to 100). byte 45: AGV battery current P: Positive N: Negative byte 46~50: AGV battery current in mA |  | TSC 🡸Loc |  |

|  | P83 |  | Contents: Robot Command |  | Direction |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Structure： byte 1: PIO direction. 0: None 1: Left 2: Right byte 2~8: From port. Can be either BUFFER1~BUFFER4 or port name byte 9~15: To port.Can be either BUFFER1~BUFFER4 or port name |  | TSC🡺Loc |  |
| S84 |  | Contents: AGV info request Ack |  | Direction |  |
|  |  | Structure： byte 1: AGV accepts or not. 0: Accept 1: From port is empty 2: To port is full |  | TSC🡺Loc |  |
| P85 |  | Contents: Move Change Request |  | Direction |  |
|  |  | Structure：28 Bytes 1Byte: 0:無需退站進站，1:到站點後執行二次定位，2:在站點執行退站 2~10Byte: P(正)N(負)P00001234為上報Routemap X:1.234，若是N00001234 X:-1.234，取小數點三位 11~19Byte: P(正)N(負)P00001234為上報Routemap Y:5.678，若是N00001234 Y:-5.678，取小數點三位 20~22Byte: 車頭方向單位為度，通常為0、90、180、270度，注意不可有負數 23~26Byte: 速度限制:500 設定值/1000，500/1000=0.5 ，若當前最高車速為1m/s 1m/0.5 = 0.5m/s 27Byte: 選擇開啟AGV槽位 0 : 無，1 : 左開蓋，2 : 右開蓋，3 : 左右開蓋 28Byte = Keep 或 GO ，Keep時該命令不會執行，AGV只會執行GO 範例: TSC請求AGVL該站將停止(Go)點改為Keep點 命令 = 2P00001234P0000567809005000K |  | TSC 🡺Loc |  |
| S86 |  | Contents: Move Change Request Ack. |  | Direction |  |
|  |  | Structure：1 Bytes 0 = NG, 1 = OK.. |  | TSC 🡸Loc |  |
| P87 |  | Contents: Robot Command |  | Direction |  |
|  |  | Structure： byte 1: PIO direction. 0: None 1: Left 2: Right <From Port>:不限長度 <To Port>:不限長度 byte 2~N From port. Can be either BUFFER1~BUFFER4 or port name Byte N: To port.Can be either BUFFER1~BUFFER4 or port name |  | TSC 🡺Loc |  |
| S88 |  | Contents: AGV info request Ack |  | Direction |  |
|  |  | Structure： byte 1: AGV accepts or not. 0: Accept 1: From port is empty 2: To port is full |  | TSC 🡸Loc |  |
| P89 |  | Contents: Erack Placement Signal |  | Direction |  |
|  |  | Structure byte 1:. 0: NG 代表可能手臂未把貨物放置好至貨架上，此命令會使手臂暫停，直到收到OK 1: OK手臂確實將貨物放置到貨架上，手臂可以回到初始位置 |  | TSC 🡺Loc |  |
| S90 |  | Contents: Erack Placement Signal Ack |  | Direction |  |
|  |  | Structure：Header only |  | TSC 🡸Loc |  |

| P91 |  | Contents: 回報目前地圖 | Direction |
| --- | --- | --- | --- |
|  |  | Structure： 1 ~ N Byte = 地圖名稱 | TSC🡸Loc |
| S92 |  | Contents: 回報目前地圖確認 | Direction |
|  |  | Structure：Header only | TSC🡺Loc |
| P93 |  | Contents: 更換地圖 | Direction |
|  |  | Structure： 1 ~ N Byte = 地圖名稱 | TSC🡺Loc |
| S94 | Contents: 更換地圖確認 |  | Direction |
|  | Structure： 1 Byte = 0:拒絕1:接受 |  | TSC🡸Loc |
| P95 | Contents: 更換地圖狀態 |  | Direction |
|  | Structure： 1 Byte = 0:失敗1:成功 |  | TSC🡸Loc |
| S96 | Contents: 更換地圖狀態確認 |  | Direction |
|  | Structure：Header only |  | TSC🡺Loc |

- Normal Process Scenario

- Online

- Move

- Load/Unload

- Abnormal Process Scenario